- Connection Manual

- Advantage 400

Delta Tau Europa AG

Delta Tau Europa AG, Gewerbestrasse 8, CH-8212 Neuhausen, Tel. 0041/526252088, Fax: 0041/526254482,

dteuropa@deltatau.com

www.deltatau.com

Mechanical Dimensions “new” enclosure

| View 2-11: Enclosure Diagram for unit of 2004 and later |  |
| --- | --- |

For fixation, these are 6x threaded stud M4 x 20 mm

Signal interfaces

All signal interfaces are found on the rear side of the control box.

Ports for the machine peripheral are female or male D-Sub connectors.

Exceptions are the IDC header for the optional MUX port (XX+)

| View 2-2 Position and name of the connectors on the control boxes rear side |  |
| --- | --- |

### Encoder connectors (ENC n)

On the backside of the control box there are connectors for 6 incremental encoders.

For encoders n (n=1-6) of the axis 1-6 there’s the following specification:

Connector type: 		see View 2 -4

Position of connector: 	see View 2 –5 and View 2 -6

Name of connector:

| View 2-3: Name of encoder connectors | Female connector; Axis; Connector. ENC 1; X; D-SUB15 Female. ENC 2; Y; D-SUB15 Female. ENC 3; Z; D-SUB15 Female. ENC 4; U / A / S; D-SUB15 Female. ENC 5/Hand Wheel; V/ B / HW; D-SUB15 Female. ENC6; free; D-SUB9 Female. |
| --- | --- |

Cable cross section:

- Signal lines>=0.14mm2

- Supply lines>=0.5mm2

Pin out of connector ENC n (n=1-5):

| View 2-4 Pin out of connector ENC1-ENC5 | Enc n. Pin#; Symbol; Function; Description. 1; CHA1+; Input; Pos. A Chan.. 2; GND; Common; Digital Reference. 3; CHB1-; Input; Neg. B Chan.. 4; CHC1+; Input; Pos. C Chan.. 5; A+5V; Output; Digital Supply. 6; NC. 7; NC. 8; NC. 9; CHA1-; Input; Neg. A Chan.. 10; CHB1+; Input; Pos. B Chan.. 11; GND; Common; Digital Reference. 12; CHC1-; Input; Neg. C Chan.. 13; NC. 14; A+5V; Output; Digital Supply. 15; NC. |
| --- | --- |

Pin out of connector ENC6:

| View 2-5 Pin out of connector ENC6 | Enc 6. Pin#; Symbol; Function; Description. 1; GND; Common; Digital Reference. 2; CHA6+; Input; Pos. A Chan.. 3; CHA6-; Input; Neg. A Chan.. 4; CHB6+; Input; Pos. B Chan.. 5; CHB6-; Input; Neg. B Chan.. 6; A+5V; Output; Digital Supply. 7; CHC6+; Input; Pos. C Chan.. 8; CHC6-; Input; Neg. C Chan.. 9; NC; .. |
| --- | --- |

Any signal type (e.g. A, A/) on the control box side is connected to a differential line driver of the following type:

| View 2-6: Encoder input driver |  |
| --- | --- |

Supported encoder types:

Incremental encoders with differentials A quad B and C Signals (RS422).

Encoder specification:

- Signal lines  A, A/, B, B/, optional C, C/

- Max. Cycle output frequency 20MHz

- Current consumption  <100mA

- Phase difference of A to B: 90 degree +/-30

Encoder power:

The control boxes supplies 5 VDC via the respective female ENC n connector.

The max. Current consumption of all consumers like the

- Encoders connected to ENC1, ENC2, ENC3, ENC4, ENC5/HW1and ENC6

- Pulse and direction drivers on STP1 and

- Multiplexed IO boards connected to XX1

Must NOT exceed 2A!

Encoder connecting cable:

Length:

The maximum cable length depends on the transmission frequency and the power consumption of the encoder:

| View 2-7: Max. Cable length of the encoder cables | Current consumption; Frequency; Max. Cable length. <100mA; 1MHz; 10m. <100mA; 2MHz; 5m. |
| --- | --- |

Only use shielded twisted pair signal lines, put shield on conductive part of connector

WARNING

ONLY CONNECT ENCODERS WHEN CONTROLLER IS POWERED DOWN.

+/- 10V drive interface (Amp1)

This interface is intended to connect 5 amplifiers with +/-10V command.

In order to use the amplifier enable lines, external 24V is needed.

- 5 differential/ single ended +/- 10V command output signals

- Analog ground used as reference for a single ended command signal only

- 5 amplifier enable lines

- 5 amplifier fault lines

Name of the connector:	AMP1

Position of connector: 	See picture 2 -9

Connector type:		female, D-Sub 25

Cable cross-section:

- Signal lines >=0.34mm2

- Power lines >=0.5mm2

Pin Out of AMP1:

| View 2-8 Pin out of AMP1 connector | AMP 1. Pin#; Symbol; Function; Description. 1; DAC1A+; Output; Analog Out + X-Axis. 2; DAC1A-; Output; Analog Out - X-Axis. 3; DAC2A+; Output; Analog Out + Y-Axis. 4; DAC2A-; Output; Analog Out - Y-Axis. 5; DAC3A+; Output; Analog Out + Z-Axis. 6; DAC3A-; Output; Analog Out - Z-Axis. 7; DAC4A+; Output; Analog Out + U / A / S. 8; DAC4A-; Output; Analog Out - U / A / S. 9; AGND; Output; Ref. for command (analog) out. 10; AMP1-ENA; Output; Amplifier Enable X-Axis. 11; AMP2-ENA; Output; Amplifier Enable Y-Axis. 12; AMP3-ENA; Output; Amplifier Enable Z-Axis. 13; AMP4-ENA; Output; Amplifier Enable U / A / S. 14; AMP5-ENA; Output; Amplifier Enable U / A / S. 15; AMP1-FLT; Input; Amplifier fault 1 X-Axis. 16; AMP2-FLT; Input; Amplifier fault 2 Y-Axis. 17; AMP3-FLT; Input; Amplifier fault 3 Z-Axis. 18; AMP4-FLT; Input; Amplifier fault 4 U / A / S-Axis. 19; AGND; Output; Ref. for command (analog) out. 20; AMP5-FLT; Amplifier fault 5 V / B-Axis. 21; DAC5A+; Analog Out + V / B-Axis. 22; DAC5A-; Analog Out - V / B-Axis. 23; NC. 24; Ext +24V; Input; External +24V. 25; Ext GND; Input; External ground, "FIELDRETURN". |
| --- | --- |

Amplifier enables:

The following block diagram shows the amp enable and its corresponding power supply:

| View 2-9: Principle of amplifier enable |  |
| --- | --- |

The specs of the amp enable lines is the following:

| View 2-10 Electrical specs of the amp enable | Parameter; Min; Max; Unit. Voltage range; 12; 24; V. Switching current; 0.5; A. |
| --- | --- |

Amp fault Inputs:

The amp fault inputs are axis specific. The reference signal for them is on AMP1 Pin 25

View 2-12 shows the amp fault input block diagram:

| View 2-11: block diagram of amplifier fault inputs |  |
| --- | --- |

The electrical specs of the amp fault is the following:

| View 2-12: electrical specs of the amp faults inputs | 1- Signal voltage range; 11-30 V. 1-Signal current consumption; 5-20mA. 0- Signal voltage range; 0-5 V or input not connected. |
| --- | --- |

Analog command outputs (+/-10V):

Singled ended command output connection

If the drive is single ended, connect only command output DAC+ to the input of the drive. Connect analog AGND (PIN 9) to the drives’ reference.

Differential command output connection

If the drive has a differential command input driver connect both, DAC+ und DAC-, to the input driver of the drive. Leave AGND floating.

| View 2-13: Schematic of command output |  |
| --- | --- |

The following chart shows the specs of the command output signals:

| View 2-14: Command output specs | Parameter; Min; Max; Units. Volage range; - 10; 10; V. Output current; -3; 3; mA. Resolution; 12 bit. |
| --- | --- |

Only use shielded twisted pair signal lines, put shield on conductive part of connector.

### Flag connector (FLG1)

This male connector has all axis specific “FLAG” inputs for 5 axes. Each axis has the following set of inputs:

- Positive end limit (+LIM)

- Negative end limit (-LIM)

- User input (User)

- Home input (HOME)

Name of connector: FLAG1

Position of connector:	See View 2 -16

Connector type:		Male, D-Sub 25

Cable cross section:

- Signal lines >=0.34mm2

- Power lines>=0.5mm2

- Pin out of FLG1:

| View 2-15: Pin out of FLAG 1 connector | FLG1. Pin#; Symbol; Function; Description. 1; USER1; Input; General Capture Flag **. 2; PLIM1; Input; Positive Limit Flag **. 3; MLIM1; Input; Negative Limit Flag **. 4; HOME1; Input; Home Flag **. 5; 24VDC out; Field power supply 24 VDC. 6; USER2; Input; General Capture Flag **. 7; PLIM2; Input; Positive Limit Flag **. 8; MLIM2; Input; Negative Limit Flag **. 9; HOME2; Input; Home Flag **. 10; 24VDC out; Field power supply 24 VDC. 11; USER3; Input; General Capture Flag **. 12; PLIM3; Input; Positive Limit Flag **. 13; MLIM3; Input; Negative Limit Flag **. 14; HOME3; Input; Home Flag **. 15; nc; Not connected. 16; USER4; Input; General Capture Flag **. 17; PLIM4; Input; Positive Limit Flag **. 18; MLIM4; Input; Negative Limit Flag **. 19; HOME4; Input; Home Flag **. 20; GND out; Output; GND field power supply, flag ref. 21; USER5; Input; General Capture Flag **. 22; PLIM5; Input; Positive Limit Flag **. 23; MLIM5; Input; Negative Limit Flag **. 24; HOME5; Input; Home Flag **. 25; GND out; Output; GND field power supply, flag ref. |
| --- | --- |

There are 2 24VDC outputs on FLG1 to supply potential sensors. Those 24 VDC lines are put through from the external field power supply.

| View 2-16 Block diagram of flag inputs |  |
| --- | --- |

The electrical specs of the flag inputs shows the following chart:

| View 2-17: 18 Electrical specs of flag inputs | 1- Signal voltage range; 11-30 V. 1-Signal current range; 5-20mA. 0- Signal voltage range; 0-5 V or input not connected. |
| --- | --- |

### Stepper connector (STP1)

This connector is used to connect up to 4 stepper drives with pulse- and direction interface. Furthermore this connector has the 4 axis specific amp fault inputs.

Per stepper axis you have the following signals:

- 1 differential pulse signal

- 1 differential direction signal

- Reference line (for all 4) for pulse and direction signals

- 1 amplifier fault line

Furthermore this connector has the outputs “EQU” for axis 1 to 4.

Per axis you have the following signal:

- 1 compare equal output

Name of the connector:		STP1

Position of the connector:		See View 2 –21a and 2 -21b

Connector type:			female, D-Sub 25

Cable cross section:

- Signal lines >=0.34mm2

- Power lines>=0.5mm2

Pin out of STP1:

| View 2-21 a: Pin out of stepper connector | STP1 (for units with S/N #3000 and later). Pin#; Symbol; Function; Description. 1; DIR_1+; Output; Direction +. 2; DIR_1-; Output; Direction -. 3; PUL_1+; Output; Pulse Output +. 4; PUL_1-; Output; Pulse Output -. 5; BEQU1; Output; EQU output. 6; PUL_5-; Output; Pulse Output - **. 7; DIR_2+; Output; Direction +. 8; DIR_2-; Output; Direction -. 9; PUL_2+; Output; Pulse Output +. 10; PUL_2-; Output; Pulse Output -. 11; BEQU2; Output; EQU output. 12; PUL_5+; Output; Pulse Output + **. 13; DIR_3+; Output; Direction +. 14; DIR_3-; Output; Direction -. 15; PUL_3+; Output; Pulse Output +. 16; PUL_3-; Output; Pulse Output -. 17; BEQU3; Output; EQU output. 18; DIR_5+; Output; Direction + **. 19; DIR_4+; Output; Direction +. 20; DIR_4-; Output; Direction -. 21; PUL_4+; Output; Pulse Output +. 22; PUL_4-; Output; Pulse Output -. 23; BEQU4; Output; EQU output. 24; DIR_5-; Output; Direction - **. 25; GND; Common; Reference for pulse and direction. |
| --- | --- |

| View 2-21 b: Pin out of stepper connector | STP1 (for units with S/N up to #2150). Pin#; Symbol; Function; Description. 1; DIR_1+; Output; Direction +. 2; DIR_1-; Output; Direction -. 3; PUL_1+; Output; Pulse Output +. 4; PUL_1-; Output; Pulse Output -. 5; BEQU1; Output; EQU output. 6; AMP1-FLT; Input; Amplifier fault **. 7; DIR_2+; Output; Direction +. 8; DIR_2-; Output; Direction -. 9; PUL_2+; Output; Pulse Output +. 10; PUL_2-; Output; Pulse Output -. 11; BEQU2; Output; EQU output. 12; AMP2-FLT; Input; Amplifier fault **. 13; DIR_3+; Output; Direction +. 14; DIR_3-; Output; Direction -. 15; PUL_3+; Output; Pulse Output +. 16; PUL_3-; Output; Pulse Output -. 17; BEQU3; Output; EQU output. 18; AMP3-FLT; Input; Amplifier fault **. 19; DIR_4+; Output; Direction +. 20; DIR_4-; Output; Direction -. 21; PUL_4+; Output; Pulse Output +. 22; PUL_4-; Output; Pulse Output -. 23; BEQU4; Output; EQU output. 24; AMP4-FLT; Input; Amplifier fault **. 25; GND; Common; Reference for pulse and direction. |
| --- | --- |

Signals to stepper interface:

The following Block-Diagram shows the pulse and direction interface:

| View 2-19: Driver RS422 for the pulse and direction signals |  |
| --- | --- |

EQU outputs:

Those output are very fast (Signal delay time 50 nsec on controller side) switching at the actual encoder position.

| View 2-20: EQU output driver |  |
| --- | --- |

The electrical specs of the EQU outputs shows the following chart:

| View 2-21: Electrical specification of the EQU output | 1- Signal voltage range; 0-2 V. 1-Signal current consumption; 5 mA. 0- Signal voltage range; 3-5 V or output open. |
| --- | --- |

The EQU outputs signal is 5V TTL,, so the user may have to design an interface circuitry.

### Analog IO connector (ANA1)

This connector has the signals for 1 analog (0 -10V) input and optional 2 inputs (+/-10V) and 2 analog outputs, in detail the following:

- 1 input 0 to 10V 12bit

- 2 differential inputs (+/-10V) 12 bit, (optional)

- 2 differential analog outputs (+/10V), In a 5-Axes configuration the first one is used for the 5th servo axis. Otherwise as general-purpose output.

Name of the connector:		ANA1

Position of the connector:		See View 2 –24a and 2 -24b

Connector type:			female, D-Sub 15

Cable cross section:

- Signal lines >=0.34mm2

- Power lines>=0.5mm2

Pin out of ANA1:

| View 2-24 a: Pin out of connector ANA1 | ANA1 (for units with S/N #3000 and later). Pin#; Symbol; Function; Description. 1; ADCIN1 +; Input +/- 10V; Analog input 1 + (optional). 2; ADCIN1 -; Input +/- 10V; Analog input 1 - (optional). 3; ADCIN2 +; Input +/- 10V; Analog input 2 + (optional). 4; ADCIN2 -; Input +/- 10V; Analog input 2 - (optional). 5; ADCIN3 +; Input 0-10V; analog Input 3 +. 6; ADCIN3 -; Input GND; Analog input 3 -. 7; A+5V; Output; Digital Supply. 8; NC; Not connected. 9; DAC5+; Output; Analog Out 5 +. 10; DAC5-; Output; Analog Out 5 -. 11; DAC6+; Output; Analog Out 6 +. 12; DAC6-; Output; Analog Out 6 -. 13; ADCIN 4 +; Input 0-5V; ** see note1. 14; ADCIN4 -; Input GND; ** see note1. 15; AAGND; Output; Reference for ADC/DAC signals. |
| --- | --- |

** note1: in case of ADV400 with front panel (not Lite and not Pack), the analog input 4 is connected in parallel with the feed override potentiometer from the front. It’s recommended not to use the ADCIN 4 (pins 13 and 14) of this connector in this case.

| View 2-24 b: Pin out of connector ANA1 | ANA1 (for units with S/N up to #2150). Pin#; Symbol; Function; Description. 1; ADCIN1 +; Input +/- 10V; Analog input 1 + (optional). 2; ADCIN1 -; Input +/- 10V; Analog input 1 - (optional). 3; ADCIN2 +; Input +/- 10V; Analog input 2 + (optional). 4; ADCIN2 -; Input +/- 10V; Analog input 2 - (optional). 5; ADCIN3 +; Input 0-10V; analog Input 3 +. 6; ADCIN3 -; Input GND; Analog input 3 -. 7; DIR_5+; Output; Direction + Axis 5. 8; PUL_5+; Output; Pulse Output + Axis 5. 9; DAC5+; Output; Analog Out 5 +. 10; DAC5-; Output; Analog Out 5 -. 11; DAC6+; Output; Analog Out 6 +. 12; DAC6-; Output; Analog Out 6 -. 13; NC; Not connected. 14; NC; Not connected. 15; AAGND; Output; Reference for ADC/DAC signals. |
| --- | --- |

Analog inputs:

The following block diagram shows the input circuitry principle of the analog inputs:

| View 2-22: Schematic of analog inputs |  |
| --- | --- |

The electrical specs of the analog inputs shows the following chart:

| View 2-23 Specs of the analog inputs | Parameter; Units/ note. Voltage range; -10-10; V. Resolution; 12; Bits. Conversion rate; Servo cycle; Default 442usec. |
| --- | --- |

Analog outputs

The following block diagram shows the circuitry of the analog outputs:

| View 2-24: Output circuitry of the analog outputs |  |
| --- | --- |

Outputs are referenced to pin15, „analog ground“

The electrical specs of the analog outputs shows the following chart:

| View 2-25: Electrical specs of the analog outputs | Parameter; Min; Max; Units. Voltage range; - 10; 10; V. Output current; -3; 3; mA. Resolution; 12 bit. |
| --- | --- |

### Digital input connector (IN1)

On this connector one can connect up to 32 optically isolated digital inputs. The inputs are grouped into 16 inputs.

Inputs 1-16 are referenced to pin36

Die inputs 17-32 are referenced to pin33

Name of the connector:		IN1

Position of the connector:		See View 2 -29

Connector type:			male, D-Sub 37

Cable cross section:

- Signal lines >=0.34mm2

- Power lines>=0.75mm2

Pin out for IN1:

| View 2-26: pin out for the digital inputs on IN1 | IN1. Pin #; Symbol; Function; Description. 1; INPUT1; Input; Digital input 1, group 1. 2; INPUT2; Input; Digital input 2, group 1. 3; INPUT3; Input; Digital input 3, group 1. 4; INPUT4; Input; Digital input 4, group 1. 5; INPUT5; Input; Digital input 5, group 1. 6; INPUT6; Input; Digital input 6, group 1. 7; INPUT7; Input; Digital input 7, group 1. 8; INPUT8; Input; Digital input 8, group 1. 9; INPUT9; Input; Digital input 9, group 1. 10; INPUT10; Input; Digital input 10, group 1. 11; INPUT11; Input; Digital input11, group 1. 12; INPUT12; Input; Digital input 12, group 1. 13; INPUT13; Input; Digital input 13, group 1. 14; INPUT14; Input; Digital input 14, group 1. 15; INPUT15; Input; Digital input 15, group 1. 16; INPUT16; Input; Digital input 16, group 1. 17; INPUT17; Input; Digital input 1, group 2. 18; INPUT18; Input; Digital input 2, group 2. 19; INPUT19; Input; Digital input 3, group 2. 20; INPUT20; Input; Digital input 4, group 2. 21; INPUT21; Input; Digital input 5, group 2. 22; INPUT22; Input; Digital input 6, group 2. 23; INPUT23; Input; Digital input 7, group 2. 24; INPUT24; Input; Digital input 8, group 2. 25; INPUT25; Input; Digital input 9, group 2. 26; INPUT26; Input; Digital input 10, group 2. 27; INPUT27; Input; Digital input11, group 2. 28; INPUT28; Input; Digital input 12, group 2. 29; INPUT29; Input; Digital input 13, group 2. 30; INPUT30; Input; Digital input 14, group 2. 31; INPUT31; Input; Digital input 15, group 2. 32; INPUT32; Input; Digital input 16, group 2. 33; I_RET2; Input; Ref for group 2 inputs. 34; NC; Not connected. 35; NC; Not connected. 36; I_RET1; Input; Ref for group 1 inputs. 37; Ext GND; Input; Ref for external field supply. |
| --- | --- |

The following block diagram in View 2-30 shows the input circuitry of the inputs:

| View 2-27:Digital input block diagram |  |
| --- | --- |

The electrical specs of the digital inputs are as follows:

| View 2-28: Electrical specs of the general digital inputs | 1- Signal voltage range; 11-30 V. 1-Signal current consumption; 5-20mA. 0- Signal voltage range; 0- 5 V or input open. |
| --- | --- |

### Digital output connector (OUT1)

The connector gives the option to connect 16 digital general-purpose outputs.

Those outputs are optically isolated, can drive up to 0.5A and are grouped into 2 sets.

The following signals are found on OUT1:

- Digital outputs 1-8

- +24V for outputs 1-8

- Digital outputs 9-16

- +24V for outputs 9-16

- 2 field power return lines

Name of the connector:		OUT1

Position of the connector:		See View 2-32

Connector type:			male, D-Sub 25

Cable cross section:

- Signal lines >=0.34mm2

- Power lines>=0.75mm2

Pin out of OUT1:

| View 2-29: Pin out of the connector for the digital outputs (OUT1) | OUT1. Pin #; Symbol; Function; Description. 1; OUTPUT1; Output; Digital output 1. 2; OUTPUT2; Output; Digital output 2. 3; OUTPUT3; Output; Digital output 3. 4; OUTPUT4; Output; Digital output 4. 5; OUTPUT5; Output; Digital output 5. 6; OUTPUT6; Output; Digital output 6. 7; OUTPUT7; Output; Digital output 7. 8; OUTPUT8; Output; Digital output 8. 9; Ext +24V; Input; External power supply 24VDC. 10; Ext GND; Input; Ref, field return- ext. Power supply. 11; NC; Not connected. 12; NC; Not connected. 13; NC; Not connected. 14; NC; Not connected. 15; NC; Not connected. 16; OUTPUT9; Output; Digital output 9. 17; OUTPUT10; Output; Digital output 10. 18; OUTPUT11; Output; Digital output 11. 19; OUTPUT12; Output; Digital output 12. 20; OUTPUT13; Output; Digital output 13. 21; OUTPUT14; Output; Digital output 14. 22; OUTPUT15; Output; Digital output 15. 23; OUTPUT16; Output; Digital output 16 * (see remark). 24; Ext +24V; Input; External +24V. 25; Ext GND; Input; External field ground. |
| --- | --- |
|  | *Output 16 is used as Drive enable in 5-Axis configuration |

The following block diagram in View 2-33 shows the output circuitry:

| View 2-30 Block diagram of the general digital outputs (OUT1) |  |
| --- | --- |

The electrical specs of the outputs shows the following chart:

| View 2-31: Electrical specs of the general digital outputs OUT1 | 1- Signal voltage range; Supply voltage. 1-Signal current consumption; Max 0.5 A, typical 5mA-0.3A. 0- Signal, max. Leakage current; 2mA. |
| --- | --- |

The power supply voltage can range between 12 and 24VDC. If all 16 outputs are powered by one power supply, the references of the 2 sets have to be connected to the supply reference. See also chapter .

for inductive loads, make sure that a quench diode in parallel to it.

### MUX IO connector (XX1)

This IDC header for MUX IO gives the option to expand the digital IO capability of the controller. Usually Delta Tau’s MUX IO boards (like ACC.34 AA) are connected here.

Only use shielded twisted pair signal lines, put shield on conductive part of connector

Name of the connector:		XX1

Position of the connector:

Connector type:			IDC header, 26 pin

## Power supplies

The ADV400 has to be supplied by at least 2 power supplies.

Besides the AC/DC- power supply to supply the internal logic power, a field power supply has to be connected to the machine peripheral.

### Internal logic power supply

The internal AC/DC power supply is already integrated into the control box and only has to be connected to the following connector:

Name of the connector:		AC POWER INPUT

Position of the connector:		See View 2-35

The following block diagram shows the AC/DC interface:

| View 2-32: Block diagram of the AC/DC interface |  |
| --- | --- |

The electrical specs of the AC/DC power supply show the following chart:

| View 2-33: Specs of the AC/DC power supply | Parameter; Range; Unit. Input voltage; 85-264; VAC. Nominal power; 40; W. Nom. Current consumption; 2; Arms. Inrush current; <18; Arms. |
| --- | --- |

### Field Voltage supply

External 12-24VDC has to be connected to maintain galvanic isolation between I/O’s and logic.

# Communication interfaces

## RS232

This port is to communicate direct with PMAC2104 from external PC, using  Delta Tau’s terminal program family  „PEWIN“

- Variables, parameter program download

- Uploads of parameters, programs for data backup

- Axis PID tuning

The serial cable should be shorter than 15m. The following cable pin out is suggested:

| View 2-37: RS232 cable |  |
| --- | --- |

HINT

DTR and DTS are hardwired on the controller side. Those signals have to be connected for  PEWIN ( PRO) communication.

## USB interface

- Supported by Windows CE

- Use to connect keyboard, mouse, floppy, etc.

- Data exchange with controller

## Ethernet (optional)

- Supported by Windows CE

- Data exchange with controller

Appendix 1: Hand Wheel box connection diagram

Connection diagram of the optional EDHW-B Hand Wheel box to ADV400.